Date Submitted: 11/19/2018

Task 00:  For this task we just had to label our launchpads. The first device listed in the image below is our Transmit while the other is our receive.

Task 01:  The goal of this task is to transmit 100 packets of data over to our receive launchpad which was done successfully as shown by the video and image below.

------------------------------------------------------------------------------------

Task 02:  For this task we had to import the rfPacketTx example project found in our simplelink sdk cc13x folder. The code below was the imported code that was responsible for transmitting packets of data over to the receive launchpad.  The image below shows the modification to the rfPacketTX’s target configuration, so it knows that the launchpad with the serial number L400A7SA will act as our transmitter.

Youtube Link: https://www.youtube.com/watch?v=aF38S6YWQdk

Modified Schematic (if applicable):

Modified Code:

// Insert code here

/***** Includes *****/

/* Standard C Libraries */

#include <stdlib.h>

#include <unistd.h>

/* TI Drivers */

#include <ti/drivers/rf/RF.h>

#include <ti/drivers/PIN.h>

#include <ti/drivers/pin/PINCC26XX.h>

/* Driverlib Header files */

#include DeviceFamily_constructPath(driverlib/rf_prop_mailbox.h)

/* Board Header files */

#include "Board.h"

#include "smartrf_settings/smartrf_settings.h"

/***** Defines *****/

/* Do power measurement */

//#define POWER_MEASUREMENT

/* Packet TX Configuration */

#define PAYLOAD_LENGTH      30

#ifdef POWER_MEASUREMENT

#define PACKET_INTERVAL     5  /* For power measurement set packet interval to 5s */

#else

#define PACKET_INTERVAL     500000  /* Set packet interval to 500000us or 500ms */

#endif

/***** Prototypes *****/

/***** Variable declarations *****/

static RF_Object rfObject;

static RF_Handle rfHandle;

/* Pin driver handle */

static PIN_Handle ledPinHandle;

static PIN_State ledPinState;

static uint8_t packet[PAYLOAD_LENGTH];

static uint16_t seqNumber;

/*

* Application LED pin configuration table:

*   - All LEDs board LEDs are off.

*/

PIN_Config pinTable[] =

{

Board_PIN_LED1 | PIN_GPIO_OUTPUT_EN | PIN_GPIO_LOW | PIN_PUSHPULL | PIN_DRVSTR_MAX,

#if defined Board_CC1352R1_LAUNCHXL

Board_DIO30_RFSW | PIN_GPIO_OUTPUT_EN | PIN_GPIO_HIGH | PIN_PUSHPULL | PIN_DRVSTR_MAX,

#endif

#ifdef POWER_MEASUREMENT

#if defined(Board_CC1350_LAUNCHXL)

Board_DIO30_SWPWR | PIN_GPIO_OUTPUT_EN | PIN_GPIO_HIGH | PIN_PUSHPULL | PIN_DRVSTR_MAX,

#endif

#endif

PIN_TERMINATE

};

/***** Function definitions *****/

void *mainThread(void *arg0)

{

RF_Params rfParams;

RF_Params_init(&rfParams);

/* Open LED pins */

ledPinHandle = PIN_open(&ledPinState, pinTable);

if (ledPinHandle == NULL)

{

while(1);

}

#ifdef POWER_MEASUREMENT

#if defined(Board_CC1350_LAUNCHXL)

/* Route out PA active pin to Board_DIO30_SWPWR */

PINCC26XX_setMux(ledPinHandle, Board_DIO30_SWPWR, PINCC26XX_MUX_RFC_GPO1);

#endif

#endif

RF_cmdPropTx.pktLen = PAYLOAD_LENGTH;

RF_cmdPropTx.pPkt = packet;

RF_cmdPropTx.startTrigger.triggerType = TRIG_NOW;

/* Request access to the radio */

rfHandle = RF_open(&rfObject, &RF_prop, (RF_RadioSetup*)&RF_cmdPropRadioDivSetup, &rfParams);

/* Set the frequency */

RF_postCmd(rfHandle, (RF_Op*)&RF_cmdFs, RF_PriorityNormal, NULL, 0);

while(1)

{

/* Create packet with incrementing sequence number and random payload */

packet[0] = (uint8_t)(seqNumber >> 8);

packet[1] = (uint8_t)(seqNumber++);

uint8_t i;

for (i = 2; i < PAYLOAD_LENGTH; i++)

{

packet[i] = rand();

}

/* Send packet */

RF_EventMask terminationReason = RF_runCmd(rfHandle, (RF_Op*)&RF_cmdPropTx,

RF_PriorityNormal, NULL, 0);

switch(terminationReason)

{

case RF_EventLastCmdDone:

// A stand-alone radio operation command or the last radio

// operation command in a chain finished.

break;

case RF_EventCmdCancelled:

// Command cancelled before it was started; it can be caused

// by RF_cancelCmd() or RF_flushCmd().

break;

case RF_EventCmdAborted:

// Abrupt command termination caused by RF_cancelCmd() or

// RF_flushCmd().

break;

case RF_EventCmdStopped:

// Graceful command termination caused by RF_cancelCmd() or

// RF_flushCmd().

break;

default:

// Uncaught error event

while(1);

}

uint32_t cmdStatus = ((volatile RF_Op*)&RF_cmdPropTx)->status;

switch(cmdStatus)

{

case PROP_DONE_OK:

// Packet transmitted successfully

break;

case PROP_DONE_STOPPED:

// received CMD_STOP while transmitting packet and finished

// transmitting packet

break;

case PROP_DONE_ABORT:

// Received CMD_ABORT while transmitting packet

break;

case PROP_ERROR_PAR:

// Observed illegal parameter

break;

case PROP_ERROR_NO_SETUP:

// Command sent without setting up the radio in a supported

// mode using CMD_PROP_RADIO_SETUP or CMD_RADIO_SETUP

break;

case PROP_ERROR_NO_FS:

// Command sent without the synthesizer being programmed

break;

case PROP_ERROR_TXUNF:

// TX underflow observed during operation

break;

default:

// Uncaught error event - these could come from the

// pool of states defined in rf_mailbox.h

while(1);

}

#ifndef POWER_MEASUREMENT

PIN_setOutputValue(ledPinHandle, Board_PIN_LED1,!PIN_getOutputValue(Board_PIN_LED1));

#endif

/* Power down the radio */

RF_yield(rfHandle);

#ifdef POWER_MEASUREMENT

/* Sleep for PACKET_INTERVAL s */

sleep(PACKET_INTERVAL);

#else

/* Sleep for PACKET_INTERVAL us */

usleep(PACKET_INTERVAL);

#endif

}

}

------------------------------------------------------------------------------------

Task 03:  The goal of this task is to export RF settings from the SmartRF studio. The image below shows the settings we used in the RF parameters for the packet Tx.

The image below shows the new set of smartrf_settings files after we exported the code. These were then saved to the project files.

Youtube Link: https://www.youtube.com/watch?v=luJDzc6Q1js

Modified Schematic (if applicable): N/A

Modified Code:

// Insert code here

No Modifications to the code were made, we only had to use the SmartRF Studio.

------------------------------------------------------------------------------------

Task 04:  For this task, all we had to do was create another instance of SmartRF studio with our RX launchpad. The image below shows the settings we used to configure the Packet RX.

Youtube Link: https://youtu.be/Mo-PQ4IHJ0U

Modified Schematic (if applicable): N/A

Modified Code:

// Insert code here

No Modifications to the code were made, we only had to use the SmartRF Studio.

------------------------------------------------------------------------------------

Task 05: In this task we had to import another project which contained the rfPacketRx code and flash our RX launchpad with it. The image below shows the new path for the new smartrf_settings files.

Youtube Link: https://www.youtube.com/watch?v=aF38S6YWQdk

Modified Schematic (if applicable):

Modified Code:

// Insert code here

/***** Includes *****/

/* Standard C Libraries */

#include <stdlib.h>

/* TI Drivers */

#include <ti/drivers/rf/RF.h>

#include <ti/drivers/PIN.h>

/* Driverlib Header files */

#include DeviceFamily_constructPath(driverlib/rf_prop_mailbox.h)

/* Board Header files */

#include "Board.h"

/* Application Header files */

#include "RFQueue.h"

#include "smartrf_settings/smartrf_settings.h"

/***** Defines *****/

/* Packet RX Configuration */

#define DATA_ENTRY_HEADER_SIZE 8  /* Constant header size of a Generic Data Entry */

#define MAX_LENGTH             30 /* Max length byte the radio will accept */

#define NUM_DATA_ENTRIES       2  /* NOTE: Only two data entries supported at the moment */

#define NUM_APPENDED_BYTES     2  /* The Data Entries data field will contain:

* 1 Header byte (RF_cmdPropRx.rxConf.bIncludeHdr = 0x1)

* Max 30 payload bytes

* 1 status byte (RF_cmdPropRx.rxConf.bAppendStatus = 0x1) */

/***** Prototypes *****/

static void callback(RF_Handle h, RF_CmdHandle ch, RF_EventMask e);

/***** Variable declarations *****/

static RF_Object rfObject;

static RF_Handle rfHandle;

/* Pin driver handle */

static PIN_Handle ledPinHandle;

static PIN_State ledPinState;

/* Buffer which contains all Data Entries for receiving data.

* Pragmas are needed to make sure this buffer is 4 byte aligned (requirement from the RF Core) */

#if defined(__TI_COMPILER_VERSION__)

#pragma DATA_ALIGN (rxDataEntryBuffer, 4);

static uint8_t

rxDataEntryBuffer[RF_QUEUE_DATA_ENTRY_BUFFER_SIZE(NUM_DATA_ENTRIES,

MAX_LENGTH,

NUM_APPENDED_BYTES)];

#elif defined(__IAR_SYSTEMS_ICC__)

#pragma data_alignment = 4

static uint8_t

rxDataEntryBuffer[RF_QUEUE_DATA_ENTRY_BUFFER_SIZE(NUM_DATA_ENTRIES,

MAX_LENGTH,

NUM_APPENDED_BYTES)];

#elif defined(__GNUC__)

static uint8_t

rxDataEntryBuffer[RF_QUEUE_DATA_ENTRY_BUFFER_SIZE(NUM_DATA_ENTRIES,

MAX_LENGTH,

NUM_APPENDED_BYTES)]

__attribute__((aligned(4)));

#else

#error This compiler is not supported.

#endif

/* Receive dataQueue for RF Core to fill in data */

static dataQueue_t dataQueue;

static rfc_dataEntryGeneral_t* currentDataEntry;

static uint8_t packetLength;

static uint8_t* packetDataPointer;

static uint8_t packet[MAX_LENGTH + NUM_APPENDED_BYTES - 1]; /* The length byte is stored in a separate variable */

/*

* Application LED pin configuration table:

*   - All LEDs board LEDs are off.

*/

PIN_Config pinTable[] =

{

Board_PIN_LED2 | PIN_GPIO_OUTPUT_EN | PIN_GPIO_LOW | PIN_PUSHPULL | PIN_DRVSTR_MAX,

#if defined Board_CC1352R1_LAUNCHXL

Board_DIO30_RFSW | PIN_GPIO_OUTPUT_EN | PIN_GPIO_HIGH | PIN_PUSHPULL | PIN_DRVSTR_MAX,

#endif

PIN_TERMINATE

};

/***** Function definitions *****/

void *mainThread(void *arg0)

{

RF_Params rfParams;

RF_Params_init(&rfParams);

/* Open LED pins */

ledPinHandle = PIN_open(&ledPinState, pinTable);

if (ledPinHandle == NULL)

{

while(1);

}

if( RFQueue_defineQueue(&dataQueue,

rxDataEntryBuffer,

sizeof(rxDataEntryBuffer),

NUM_DATA_ENTRIES,

MAX_LENGTH + NUM_APPENDED_BYTES))

{

/* Failed to allocate space for all data entries */

while(1);

}

/* Modify CMD_PROP_RX command for application needs */

/* Set the Data Entity queue for received data */

RF_cmdPropRx.pQueue = &dataQueue;

/* Discard ignored packets from Rx queue */

RF_cmdPropRx.rxConf.bAutoFlushIgnored = 1;

/* Discard packets with CRC error from Rx queue */

RF_cmdPropRx.rxConf.bAutoFlushCrcErr = 1;

/* Implement packet length filtering to avoid PROP_ERROR_RXBUF */

RF_cmdPropRx.maxPktLen = MAX_LENGTH;

RF_cmdPropRx.pktConf.bRepeatOk = 1;

RF_cmdPropRx.pktConf.bRepeatNok = 1;

/* Request access to the radio */

rfHandle = RF_open(&rfObject, &RF_prop,

(RF_RadioSetup*)&RF_cmdPropRadioDivSetup, &rfParams);

/* Set the frequency */

RF_postCmd(rfHandle, (RF_Op*)&RF_cmdFs, RF_PriorityNormal, NULL, 0);

/* Enter RX mode and stay forever in RX */

RF_EventMask terminationReason = RF_runCmd(rfHandle, (RF_Op*)&RF_cmdPropRx,

RF_PriorityNormal, &callback,

RF_EventRxEntryDone);

switch(terminationReason)

{

case RF_EventLastCmdDone:

// A stand-alone radio operation command or the last radio

// operation command in a chain finished.

break;

case RF_EventCmdCancelled:

// Command cancelled before it was started; it can be caused

// by RF_cancelCmd() or RF_flushCmd().

break;

case RF_EventCmdAborted:

// Abrupt command termination caused by RF_cancelCmd() or

// RF_flushCmd().

break;

case RF_EventCmdStopped:

// Graceful command termination caused by RF_cancelCmd() or

// RF_flushCmd().

break;

default:

// Uncaught error event

while(1);

}

uint32_t cmdStatus = ((volatile RF_Op*)&RF_cmdPropRx)->status;

switch(cmdStatus)

{

case PROP_DONE_OK:

// Packet received with CRC OK

break;

case PROP_DONE_RXERR:

// Packet received with CRC error

break;

case PROP_DONE_RXTIMEOUT:

// Observed end trigger while in sync search

break;

case PROP_DONE_BREAK:

// Observed end trigger while receiving packet when the command is

// configured with endType set to 1

break;

case PROP_DONE_ENDED:

// Received packet after having observed the end trigger; if the

// command is configured with endType set to 0, the end trigger

// will not terminate an ongoing reception

break;

case PROP_DONE_STOPPED:

// received CMD_STOP after command started and, if sync found,

// packet is received

break;

case PROP_DONE_ABORT:

// Received CMD_ABORT after command started

break;

case PROP_ERROR_RXBUF:

// No RX buffer large enough for the received data available at

// the start of a packet

break;

case PROP_ERROR_RXFULL:

// Out of RX buffer space during reception in a partial read

break;

case PROP_ERROR_PAR:

// Observed illegal parameter

break;

case PROP_ERROR_NO_SETUP:

// Command sent without setting up the radio in a supported

// mode using CMD_PROP_RADIO_SETUP or CMD_RADIO_SETUP

break;

case PROP_ERROR_NO_FS:

// Command sent without the synthesizer being programmed

break;

case PROP_ERROR_RXOVF:

// RX overflow observed during operation

break;

default:

// Uncaught error event - these could come from the

// pool of states defined in rf_mailbox.h

while(1);

}

while(1);

}

void callback(RF_Handle h, RF_CmdHandle ch, RF_EventMask e)

{

if (e & RF_EventRxEntryDone)

{

/* Toggle pin to indicate RX */

PIN_setOutputValue(ledPinHandle, Board_PIN_LED2,

!PIN_getOutputValue(Board_PIN_LED2));

/* Get current unhandled data entry */

currentDataEntry = RFQueue_getDataEntry();

/* Handle the packet data, located at &currentDataEntry->data:

* - Length is the first byte with the current configuration

* - Data starts from the second byte */

packetLength      = *(uint8_t*)(&currentDataEntry->data);

packetDataPointer = (uint8_t*)(&currentDataEntry->data + 1);

/* Copy the payload + the status byte to the packet variable */

memcpy(packet, packetDataPointer, (packetLength + 1));

RFQueue_nextEntry();

}

}

------------------------------------------------------------------------------------

Task 06:  The goal of this task is to just let the two launchpads communicate with each other. This is demonstrated in the video linked below.

Youtube Link: https://www.youtube.com/watch?v=aF38S6YWQdk

Modified Schematic (if applicable):

Modified Code:

No Modifications to the code were made, we only had to change the settings in SmartRF Studio.

------------------------------------------------------------------------------------

Task 07:  For this task, we had to transmit packets of data using the SmartRF studio and receive those packets of data with the firmware RX. The image below shows the settings used for RF parameters and the Packet TX tab. One of the launchpads will have no blinking LED because it is not used in this task since the SmartRF studio will be the one transmitting while the one launchpad receives.

Youtube Link: https://youtu.be/9pmSrILy_Ms

Modified Schematic (if applicable): N/A

Modified Code:

No Modifications to the code were made, we only had to change the settings in SmartRF Studio.

------------------------------------------------------------------------------------